# Qreasee: Which Authoring Path to Take

Producing preliminary drawing sets like the Revit-made Alfia Wardah PDF, for 3–10 person studios

# The short answer

The SketchUp vs Revit question is really two separate choices:

- How the user draws (input feel): lines and planes, or placing predefined walls and floors.

- What the model stores (data): loose geometry, or objects like Wall, Room, Door, Slab.

Recommendation: draw like SketchUp, store like Revit. Snaptrude works roughly this way as well, as far as I know.

# Why not pure SketchUp

SketchUp stores loose faces and edges, so it has no real idea what a wall or a room is. That is the cause of the SketchUp → AutoCAD round trip: to get a plan, section or dimension out of it, someone has to redraw it or export linework and clean it up by hand. If Qreasee copies that model, it inherits the same problem.

# Why not pure Revit

Revit’s families, parameters, worksharing and “borrow elements” are what make it heavy and hard to learn. A studio of 3–10 people only needs about ten object types: Grid, Level, Room, Wall, Opening (door/window), Slab, Roof, Stair, plus 2D annotation (dimension, tag, text).

# What the PDF actually is

The Alfia Wardah PDF is not a model. It is a set of views of one model, placed on sheets:

- Grid bubbles A–F and 1–3, with chain and overall dimension strings

- Level markers (FFL 0 / 3600 / 7000) and section marks that point to sheet numbers (A105, A106)

- Room labels, stairs, a title block, and colour fills

- Plans at 1:100, the front elevation at 1:50

So Qreasee does not need to copy Revit’s way of drawing. It needs a single model, with every view generated from it and every view editable. That is also what makes Google Sheets and Canva-style collaboration possible: everyone edits objects with simple properties, not a heavy file.

# Where Qreasee stands today

From PROGRESS.md and DECISIONS.md, the hard parts already exist: the op log, stable element IDs, branches, the Studio Kit with title-block fields, and plan/section/elevation SVG generation.

The gap is that the schematic layer (rooms, walls, openings, roof) is derived from the mass. Rooms come from a guillotine partition, openings are auto-placed, and drawings cover one level only. The architect cannot really author the plan, and a plan like the Alfia house (two floors, stairs, different room sizes) cannot be produced yet.

The notes already say B-01/B-02 (making schematic entities first-class) must come before Phase C. That should be the next big step, because sheets, collaboration and handoff all depend on it.

# The chain to build

Sketch/massing → Rooms (from the program table) → Walls/openings/slabs/roof as editable objects → Views → Sheets

- Rooms from the brief become the primary object, as in Snaptrude. Drawing a room generates its walls, and moving a wall resizes the room.

- “Generate from mass” becomes only a starting point. Once the architect edits a wall, regeneration must never overwrite it. The stable IDs already support this.

- Dimensions, grid bubbles, level markers, section marks and room tags are generated from the model, with manual override.

- Sheets become real: A3 or A1 sheets with the title block from the Studio Kit, several views per sheet at a fixed scale, and vector PDF export (not PNG).

- Multi-level plans and sections come next, with stairs, because sheets A105/A106 need them.

# A concrete acceptance test

Can one person recreate sheets A101–A106 for a new house in Qreasee in one sitting, with no SketchUp or AutoCAD? This is much more specific than “replace SketchUp.”

# How the other startups compare

Based on general knowledge; check their current products before copying anything.

| Product | Approach |
| --- | --- |
| Snaptrude | Closest to the goal: program first, rooms with areas, then geometry, in a browser. |
| Hypar | More generative and code-driven, aimed at larger buildings. |
| Qonic | More about handling BIM and IFC models in the cloud. |

Qreasee’s differentiators: Canva-like onboarding, Indonesian rule presets (KRK, KDB, KLB, KDH), decision branches with recorded rationale, and the Studio Kit. Do not try to out-BIM them, and leave IFC export for later.

# A possible wedge for later

The Alfia sheet is an existing-building survey. Studios sell this as a service, and it uses the same objects (tracing over a photo or scan, plus typed dimensions). It could be a useful wedge, but get the new-design path working first.